Food u Beverage Catalogue Stainless Steel - Filter Housings

Introduction

Sartorius Stedim Biotech FuB filter housings were developed for sanitary beverage filtration applications, including:

- Beer

- Wine u spirits

- Mineral | Bottled water

- Carbonated Soft drinks

- Non-carbonated Soft drinks

Thanks to their optimized design, our FuB housings are exceptionally easy to clean for thorough results and can be in-line steam sterilized as a complete unit, incl. gaskets.

Our housings have been designed in accordance with the European Pressure Equipment Directive PED 2014/68/EU - rule EN13445.

The complete series of 810 and 813 housings are available in sizes ranging from 1 to 30 cartridges and heights from 10“ to 40“.

The cartridge socket adapter is available in code 25 (code 7) and code 28 (810 liquid u 813 liquid).

Sartorius Stedim Biotech FuB housings can be equipped with

a wide range of Inlet and Outlet connections such as DIN11851 and sanitary Clamp. Other connections are available upon request. Vent and drain options are also available in multiple designs and configurations.

Gaskets are offered in various material types depending on

the application. As standard, we use FDA, USP Class VI, EC 1935/2004 arti- cle 3 - u Phthalate-free compliant Silicone u EPDM

elastomers.

Certification is according to PED 2014/68/EU. Additional certificates like Ra roughness or material certificates (MTR‘s) are available upon request.

Only 316L grade stainless steel is used for all wetted surfaces to provide maximum durability and resistance to corrosion.

| Contents Directory |  |  |  |
| --- | --- | --- | --- |
| Introduction | 2 |  |  |
| Description | 3 |  |  |
| Contents Directory | 4 |  |  |
| Type 810 (10 bar Version) |  |  |  |
| Standard Liquid Housing T-Type - Series VHP |  |  |  |
| 1-round – incl. Spare Parts | 6 - 9 |  |  |
| 3-round – incl. Spare Parts | 10 - 13 |  |  |
| 5-round – incl. Spare Parts | 14 - 17 |  |  |
| 8-round – incl. Spare Parts | 18 - 21 |  |  |
| 12-round – incl. Spare Parts | 22 - 25 |  |  |
| 18-round – incl. Spare Parts | 26 - 29 |  |  |
| 24-round – incl. Spare Parts | 30 - 34 |  |  |
| 30-round – incl. Spare Parts | 35 - 38 |  |  |
| 36-round – incl. Spare Parts | 39 - 40 |  |  |
| Type 810 Fluid group 1 (dangerous) |  |  |  |
| Standard Liquid Housing T-Type - Series SH |  |  |  |
| 1-round – incl. Spare Parts 3-round – incl. Spare Parts | 41 - 45 46 - 49 |  |  |
| 5-round – incl. Spare Parts | 50 - 53 |  |  |
| 8-round – incl. Spare Parts | 54 - 57 |  |  |
| 12-round – incl. Spare Parts | 58 - 61 |  |  |
| 18-round – incl. Spare Parts | 62 - 65 |  |  |
| 24-round – incl. Spare Parts | 66 - 69 |  |  |
| 30-round – incl. Spare Parts | 70 - 73 |  |  |

Standard Liquid Housing In-Line - Series ALN

- round 74 -	75

Standard 1-round Gas Housing - Series KMC	76

- round 77 -	80

Type 805 (5 bar Version)	81

Standard Multi-round Gas Housing - Series AIR	82 - 84

3- | 5- | 8-round

Type 808 (8 bar Version)	85

Industrial - 1-round - liquid housing - Series FI	86 - 87

Type 800 (Pressureless)	88

Vent Housings - Series VSN

1- | 3- | 5- | 8-round	89 - 91

Jumbo 1-round (Sartorius)	92

Overview of housing types

Type 800 - Vent Housings - pressureless Cartridge number: 01 | 03 | 05 | 08

Type 805 - Gas Housings (Multi-round) - 5bar Cartridge number: 03 | 05 | 08

Type 810 - Gas Housing* | Liquid Housings - 10 bar Cartridge number: 01 | 03 | 05 | 08

*Gas Housings 10 bar only available in 1-round

Type 813 	13 bar Version

Working pressure:	max.13 bar | 189 psi Temperature:	 max. 40°C | 104°F

Working pressure:	 max.4 bar | 58 psi Temperature:	max. 150°C | 302°F

Cartridge adapter	25 | Code 7

Internal surface roughness: Ra<0.8μm | 32μin External surface roughness Ra<1.6μm | 64 μin Mechanically polished

# Type 810

## (10 bar Version)

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 01 x 10“ | 360 | 312 | 208 | 790 | 300 | 3.0 | 4.3 |
| 01 x 20“ | 360 | 312 | 208 | 1040 | 550 | 4.8 | 4.3 |
| 01 x 30“ | 360 | 312 | 208 | 1290 | 800 | 6.8 | I |
| 01 x 40“ | 360 | 312 | 208 | 1540 | 1050 | 8.6 | I |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 1.0“ (25.4mm x 1.65mm) | 381 | 188 | 103 | 381 | 269 | 154 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN25 | 381 | 220 | 118 | 381 | 300 | 169 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.0“ (25.4mm x 1.65mm) | 381 | 163 | 90 | 381 | 244 | 141 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 01 x 10“ | 2.0 | 7.0 | 65x27x17 | 8.0 |
| 01 x 20“ | 3.3 | 8.2 | 87x27x17 | 9.5 |
| 01 x 30“ | 4.6 | 9.4 | 112x27x17 | 11.0 |
| 01 x 40“ | 5.9 | 10.6 | 137x27x17 | 13.0 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.0“ |
|  | 01 | 10“ | DIN 11851 Male Part | DN 25 | Silicone | x | x |
|  | 01 | 10“ | DIN 11851 Male Part | DN 25 | EPDM | x | x |
|  | 01 | 10“ | DIN 11851 Male Part | DN 25 | Silicone | x | - |
|  | 01 | 10“ | DIN 11851 Male Part | DN 25 | EPDM | X | - |
|  | 01 | 10“ | TC | 1“ | Silicone | x | x |
|  | 01 | 10“ | TC | 1“ | EPDM | x | x |
|  | 01 | 10“ | TC | 1“ | Silicone | x | - |
|  | 01 | 10“ | TC | 1“ | EPDM | x | - |
|  | 01 | 10“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 10“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | x |
|  | 01 | 10“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | - |
|  | 01 | 10“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | - |
|  | 01 | 20“ | DIN 11851 Male Part | DN 25 | Silicone | x | x |
|  | 01 | 20“ | DIN 11851 Male Part | DN 25 | EPDM | x | x |
|  | 01 | 20“ | DIN 11851 Male Part | DN 25 | Silicone | x | - |
|  | 01 | 20“ | DIN 11851 Male Part | DN 25 | EPDM | x | - |
|  | 01 | 20“ | TC | 1“ | Silicone | x | x |
|  | 01 | 20“ | TC | 1“ | EPDM | x | x |
|  | 01 | 20“ | TC | 1“ | Silicone | x | - |
|  | 01 | 20“ | TC | 1“ | EPDM | x | - |
|  | 01 | 20“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 20“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | x |
|  | 01 | 20“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | - |
|  | 01 | 20“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | - |
|  | 01 | 30“ | DIN 11851 Male Part | DN 25 | Silicone | x | x |
|  | 01 | 30“ | DIN 11851 Male Part | DN 25 | EPDM | x | x |
|  | 01 | 30“ | DIN 11851 Male Part | DN 25 | Silicone | x | - |
|  | 01 | 30“ | DIN 11851 Male Part | DN 25 | EPDM | x | - |
|  | 01 | 30“ | TC | 1“ | Silicone | x | x |
|  | 01 | 30“ | TC | 1“ | EPDM | x | x |
|  | 01 | 30“ | TC | 1“ | Silicone | x | - |
|  | 01 | 30“ | TC | 1“ | EPDM | x | - |
|  | 01 | 30“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 30“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | x |
|  | 01 | 30“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | - |
|  | 01 | 30“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | - |
|  | 01 | 40“ | DIN 11851 Male Part | DN 25 | Silicone | x | x |
|  | 01 | 40“ | DIN 11851 Male Part | DN 25 | EPDM | x | x |
|  | 01 | 40“ | DIN 11851 Male Part | DN 25 | Silicone | x | - |
|  | 01 | 40“ | DIN 11851 Male Part | DN 25 | EPDM | x | - |
|  | 01 | 40“ | TC | 1“ | Silicone | x | x |
|  | 01 | 40“ | TC | 1“ | EPDM | x | x |
|  | 01 | 40“ | TC | 1“ | Silicone | x | - |
|  | 01 | 40“ | TC | 1“ | EPDM | x | - |
|  | 01 | 40“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 40“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | x |
|  | 01 | 40“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | - |
|  | 01 | 40“ | Welding Stubs | 25.4 x 1.65 | EPDM | x | - |

XX = 25 (Adapter 25 / code 7)	8

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | CLAMP 4“ | AISI 304 |  | 1 |
| 3a | CLAMP | AISI 316L |  | 1 |
| 3 | CARFTRIDGES PLATE + BENDS | AISI 316L |  | 1 |
| 4 | GASKET CLAMP 4“ | SILICONE |  | 1 |
|  | GASKET CLAMP 4“ | EPDM |  | 1 |
| 5 | LEG | AISI 304 |  | 3 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 03 x 10“ | 357 | 310 | 206 | 816 | 290 | 7.3 | I |
| 03 x 20“ | 357 | 310 | 206 | 1065 | 540 | 12.6 | I |
| 03 x 30“ | 357 | 310 | 206 | 1314 | 790 | 17.8 | I |
| 03 x 40“ | 357 | 310 | 206 | 1564 | 1040 | 23.1 | II |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 310 | 147 | 345 | 385 | 223 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN40 | 345 | 351 | 168 | 345 | 426 | 243 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 285 | 135 | 345 | 360 | 210 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 03 x 10“ | 4.0 | 17.4 | 80x30x37 | 23.0 |
| 03 x 20“ | 6.0 | 19.4 | 104x30x37 | 25.0 |
| 03 x 30“ | 8.0 | 21.4 | 130x30x37 | 27.0 |
| 03 x 40“ | 10.0 | 23.4 | 155x30x37 | 29.0 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 03 | 20“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 03 | 20“ | DIN 11851 Male Part | DN 40 | EPDM | x | x |
|  | 03 | 20“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 03 | 20“ | DIN 11851 Male Part | DN 40 | EPDM | x | - |
|  | 03 | 20“ | TC | 1.5“ | Silicone | x | x |
|  | 03 | 20“ | TC | 1.5“ | EPDM | x | x |
|  | 03 | 20“ | TC | 1.5“ | Silicone | x | - |
|  | 03 | 20“ | TC | 1.5“ | EPDM | x | - |
|  | 03 | 20“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 03 | 20“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | x |
|  | 03 | 20“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 03 | 20“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | - |
|  | 03 | 30“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 03 | 30“ | DIN 11851 Male Part | DN 40 | EPDM | x | x |
|  | 03 | 30“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 03 | 30“ | DIN 11851 Male Part | DN 40 | EPDM | x | - |
|  | 03 | 30“ | TC | 1.5“ | Silicone | x | x |
|  | 03 | 30“ | TC | 1.5“ | EPDM | x | x |
|  | 03 | 30“ | TC | 1.5“ | Silicone | x | - |
|  | 03 | 30“ | TC | 1.5“ | EPDM | x | - |
|  | 03 | 30“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 03 | 30“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | x |
|  | 03 | 30“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 03 | 30“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | - |
|  | 03 | 40“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 03 | 40“ | DIN 11851 Male Part | DN 40 | EPDM | x | x |
|  | 03 | 40“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 03 | 40“ | DIN 11851 Male Part | DN 40 | EPDM | x | - |
|  | 03 | 40“ | TC | 1.5“ | Silicone | x | x |
|  | 03 | 40“ | TC | 1.5“ | EPDM | x | x |
|  | 03 | 40“ | TC | 1.5“ | Silicone | x | - |
|  | 03 | 40“ | TC | 1.5“ | EPDM | x | - |
|  | 03 | 40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 03 | 40“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | x |
|  | 03 | 40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 03 | 40“ | Welding Stubs | 38.1 x 1.65 | EPDM | x | - |

XX = 25 (Adapter 25 / code 7)	12

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=158.11 | SILICONE |  | 1 |
| 5 | GASKET CLAMP 1.5“ | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L |  | 1 |
| 8 | OUTLET BEND | AISI 316L |  | 1 |
| 9 | COMPLETE LEG | AISI 304 |  | 3 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 05 x 10“ | 400 | 347 | 231 | 825 | 290 | 11 | I |
| 05 x 20“ | 400 | 347 | 231 | 1075 | 540 | 20 | II |
| 05 x 30“ | 400 | 347 | 231 | 1325 | 790 | 29.1 | II |
| 05 x 40“ | 400 | 347 | 231 | 1575 | 1040 | 38.2 | II |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 2.0“ (50.8mm x 1.65mm) | 315 | 378 | 196 | 315 | 445 | 263 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN50 | 315 | 423 | 218 | 315 | 490 | 285 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.0“ (50.8mm x 1.65mm) | 315 | 353 | 183 | 315 | 420 | 250 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 05 x 10“ | 5.3 | 11.0 | 72x35x37 | 24.0 |
| 05 x 20“ | 7.5 | 20.0 | 97x35x37 | 26.0 |
| 05 x 30“ | 10.2 | 22.0 | 122x35x37 | 28.0 |
| 05 x 40“ | 12.9 | 24.0 | 147x35x37 | 30.0 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 05 | 20“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 05 | 20“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 05 | 20“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 05 | 20“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 05 | 20“ | TC | 2“ | Silicone | x | x |
|  | 05 | 20“ | TC | 2“ | EPDM | x | x |
|  | 05 | 20“ | TC | 2“ | Silicone | x | - |
|  | 05 | 20“ | TC | 2“ | EPDM | x | - |
|  | 05 | 20“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 05 | 20“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 05 | 20“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 05 | 20“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |
|  | 05 | 30“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 05 | 30“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 05 | 30“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 05 | 30“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 05 | 30“ | TC | 2“ | Silicone | x | x |
|  | 05 | 30“ | TC | 2“ | EPDM | x | x |
|  | 05 | 30“ | TC | 2“ | Silicone | x | - |
|  | 05 | 30“ | TC | 2“ | EPDM | x | - |
|  | 05 | 30“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 05 | 30“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 05 | 30“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 05 | 30“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |
|  | 05 | 40“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 05 | 40“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 05 | 40“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 05 | 40“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 05 | 40“ | TC | 2“ | Silicone | x | x |
|  | 05 | 40“ | TC | 2“ | EPDM | x | x |
|  | 05 | 40“ | TC | 2“ | Silicone | x | - |
|  | 05 | 40“ | TC | 2“ | EPDM | x | - |
|  | 05 | 40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 05 | 40“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 05 | 40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 05 | 40“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |

XX = 25 (Adapter 25 / code 7)	16

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=208.91 | SILICONE |  | 1 |
| 5 | GASKET CLAMP 2“ | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L |  | 1 |
| 8 | OUTLET BEND | AISI 316L |  | 1 |
| 9 | COMPLETE LEG | AISI 304 |  | 3 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 08 x 10“ | 448 | 388 | 259 | 846 | 290 | 21.3 | II |
| 08 x 20“ | 448 | 388 | 259 | 1096 | 540 | 35.5 | II |
| 08 x 30“ | 448 | 388 | 259 | 1346 | 790 | 48.7 | II |
| 08 x 40“ | 448 | 388 | 259 | 1596 | 1040 | 63.9 | II |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 2.0“ (50.8mm x 1.65mm) | 315 | 404 | 222 | 315 | 471 | 289 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN50 | 315 | 449 | 244 | 315 | 516 | 311 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.0“ (50.8mm x 1.65mm) | 315 | 379 | 209 | 315 | 446 | 273 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 08 x 10“ | 7.0 | 27.0 | 75x40x42 | 30.0 |
| 08 x 20“ | 10.0 | 30.0 | 100x40x42 | 34.0 |
| 08 x 30“ | 13.0 | 3.0 | 125x40x42 | 38.0 |
| 08 x 40“ | 16.0 | 36.0 | 150x40x42 | 42.0 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 08 | 20“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 08 | 20“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 08 | 20“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 08 | 20“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 08 | 20“ | TC | 2“ | Silicone | x | x |
|  | 08 | 20“ | TC | 2“ | EPDM | x | x |
|  | 08 | 20“ | TC | 2“ | Silicone | x | - |
|  | 08 | 20“ | TC | 2“ | EPDM | x | - |
|  | 08 | 20“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 08 | 20“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 08 | 20“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 08 | 20“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |
|  | 08 | 30“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 08 | 30“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 08 | 30“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 08 | 30“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 08 | 30“ | TC | 2“ | Silicone | x | x |
|  | 08 | 30“ | TC | 2“ | EPDM | x | x |
|  | 08 | 30“ | TC | 2“ | Silicone | x | - |
|  | 08 | 30“ | TC | 2“ | EPDM | x | - |
|  | 08 | 30“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 08 | 30“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 08 | 30“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 08 | 30“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |
|  | 08 | 40“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 08 | 40“ | DIN 11851 Male Part | DN 50 | EPDM | x | x |
|  | 08 | 40“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 08 | 40“ | DIN 11851 Male Part | DN 50 | EPDM | x | - |
|  | 08 | 40“ | TC | 2“ | Silicone | x | x |
|  | 08 | 40“ | TC | 2“ | EPDM | x | x |
|  | 08 | 40“ | TC | 2“ | Silicone | x | - |
|  | 08 | 40“ | TC | 2“ | EPDM | x | - |
|  | 08 | 40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 08 | 40“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | x |
|  | 08 | 40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 08 | 40“ | Welding Stubs | 50.8 x 1.65 | EPDM | x | - |

XX = 25 (Adapter 25 / code 7)	20

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=266.06 | SILICONE |  | 1 |
| 5 | GASKET CLAMP | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L |  | 1 |
| 8 | OUTLET BEND | AISI 316L |  | 1 |
| 9 | COMPLETE LEG | AISI 304 |  | 3 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 12 x 10“ | 425 | 410 | 279 | 930 | 290 | 41 | II |
| 12 x 20“ | 425 | 410 | 279 | 1180 | 540 | 66 | II |
| 12 x 30“ | 425 | 410 | 279 | 1430 | 790 | 88 | II |
| 12 x 40“ | 425 | 410 | 279 | 1680 | 1040 | 112 | III |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 2.5“ (63.5mm x 1.65mm) | 326 | 493 | 225 | 326 | 573 | 225 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN65 | 326 | 547 | 252 | 326 | 627 | 252 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 326 | 467 | 212 | 326 | 547 | 212 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |  |
| 12 x 10“ | 10.0 | 58.0 | 75x53x54 | 63.0 |  |
| 12 x 20“ | 14.5 | 62.0 | 100x53x54 | 67.0 |  |
| 12 x 30“ | 19.0 | 66.0 | 125x53x54 | 71.0 |  |
| 12 x 40“ | 23.5 | 70.0 | 150x53x54 | 76.0 |  |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 12 | 30“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 12 | 30“ | DIN 11851 Male Part | DN 65 | EPDM | x | x |
|  | 12 | 30“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 12 | 30“ | DIN 11851 Male Part | DN 65 | EPDM | x | - |
|  | 12 | 30“ | TC | 2.5“ | Silicone | x | x |
|  | 12 | 30“ | TC | 2.5“ | EPDM | x | x |
|  | 12 | 30“ | TC | 2.5“ | Silicone | x | - |
|  | 12 | 30“ | TC | 2.5“ | EPDM | x | - |
|  | 12 | 30“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 12 | 30“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | x |
|  | 12 | 30“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 12 | 30“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | - |
|  | 12 | 40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 12 | 40“ | DIN 11851 Male Part | DN 65 | EPDM | x | x |
|  | 12 | 40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 12 | 40“ | DIN 11851 Male Part | DN 65 | EPDM | x | - |
|  | 12 | 40“ | TC | 2.5“ | Silicone | x | x |
|  | 12 | 40“ | TC | 2.5“ | EPDM | x | x |
|  | 12 | 40“ | TC | 2.5“ | Silicone | x | - |
|  | 12 | 40“ | TC | 2.5“ | EPDM | x | - |
|  | 12 | 40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 12 | 40“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | x |
|  | 12 | 40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 12 | 40“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | - |

XX = 25 (Adapter 25 / code 7)	24

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=342.30 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 3 |
| 6 | FOOT | AISI 304 |  | 3 |
| 7 | CLAMP 3“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 3“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=100.00 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 18 x 10“ | 505 | 480 | 304 | 950 | 290 | 54 | II |
| 18 x 20“ | 505 | 480 | 304 | 1200 | 540 | 85 | II |
| 18 x 30“ | 505 | 480 | 304 | 1450 | 790 | 115 | III |
| 18 x 40“ | 505 | 480 | 304 | 1700 | 1040 | 146 | III |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 2.5“ (63.5mm x 1.65mm) [Standard] | 325 | 509 | 225 | 325 | 589 | 225 |
| 3.0“ (76.2mm x 1.65mm) | 297 | 565 | 253 | 297 | 645 | 253 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN65 [Standard] | 325 | 564 | 252 | 325 | 644 | 252 |
| DN80 | 316 | 620 | 276 | 316 | 700 | 276 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) [Standard] | 325 | 482 | 212 | 325 | 565 | 212 |
| 3.0“ (76.2mm x 1.65mm) | 297 | 540 | 240 | 297 | 620 | 240 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 18 x 10“ | 12.0 | 59.0 | 78x58x59 | 64.0 |
| 18 x 20“ | 17.0 | 63.0 | 103x58x59 | 68.0 |
| 18 x 30“ | 22.0 | 68.0 | 128x58x59 | 74.0 |
| 18 x 40“ | 27.0 | 74.0 | 153x58x59 | 80.0 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ | Standard Dimension |
|  | 18 | 30“ | DIN 11851 Male Part | DN 65 | Silicone | x | x | x |
|  | 18 | 30“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |  |
|  | 18 | 30“ | DIN 11851 Male Part | DN 65 | EPDM | x | x | x |
|  | 18 | 30“ | DIN 11851 Male Part | DN 80 | EPDM | x | x |  |
|  | 18 | 30“ | DIN 11851 Male Part | DN 65 | Silicone | x | - | x |
|  | 18 | 30“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |  |
|  | 18 | 30“ | DIN 11851 Male Part | DN 65 | EPDM | x | - | x |
|  | 18 | 30“ | DIN 11851 Male Part | DN 80 | EPDM | x | - |  |
|  | 18 | 30“ | TC | 2.5“ | Silicone | x | x | x |
|  | 18 | 30“ | TC | 3.0“ | Silicone | x | x |  |
|  | 18 | 30“ | TC | 2.5“ | EPDM | x | x | x |
|  | 18 | 30“ | TC | 3.0“ | EPDM | x | x |  |
|  | 18 | 30“ | TC | 2.5“ | Silicone | x | - | x |
|  | 18 | 30“ | TC | 3.0“ | Silicone | x | - |  |
|  | 18 | 30“ | TC | 2.5“ | EPDM | x | - | x |
|  | 18 | 30“ | TC | 3.0“ | EPDM | x | - |  |
|  | 18 | 30“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x | x |
|  | 18 | 30“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |  |
|  | 18 | 30“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | x | x |
|  | 18 | 30“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | x |  |
|  | 18 | 30“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - | x |
|  | 18 | 30“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |  |
|  | 18 | 30“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | - | x |
|  | 18 | 30“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | - |  |
|  | 18 | 40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x | x |
|  | 18 | 40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |  |
|  | 18 | 40“ | DIN 11851 Male Part | DN 65 | EPDM | x | x | x |
|  | 18 | 40“ | DIN 11851 Male Part | DN 80 | EPDM | x | x |  |
|  | 18 | 40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - | x |
|  | 18 | 40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |  |
|  | 18 | 40“ | DIN 11851 Male Part | DN 65 | EPDM | x | - | x |
|  | 18 | 40“ | DIN 11851 Male Part | DN 80 | EPDM | x | - |  |
|  | 18 | 40“ | TC | 2.5“ | Silicone | x | x | x |
|  | 18 | 40“ | TC | 3.0“ | Silicone | x | x |  |
|  | 18 | 40“ | TC | 2.5“ | EPDM | x | x | x |
|  | 18 | 40“ | TC | 3.0“ | EPDM | x | x |  |
|  | 18 | 40“ | TC | 2.5“ | Silicone | x | - | x |
|  | 18 | 40“ | TC | 3.0“ | Silicone | x | - |  |
|  | 18 | 40“ | TC | 2.5“ | EPDM | x | - | x |
|  | 18 | 40“ | TC | 3.0“ | EPDM | x | - |  |
|  | 18 | 40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x | x |
|  | 18 | 40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |  |
|  | 18 | 40“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | x | x |
|  | 18 | 40“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | x |  |
|  | 18 | 40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - | x |
|  | 18 | 40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |  |
|  | 18 | 40“ | Welding Stubs | 63.5 x 1.65 | EPDM | x | - | x |
|  | 18 | 40“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | - |  |

XX = 25 (Adapter 25 / code 7)	28

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=393.10 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 3 |
| 6 | FOOT | AISI 304 |  | 3 |
| 7 | CLAMP 3“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 3“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=110.50 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 24 x 10“ | 465 | 455 | - | 970 | 290 | 73 | II |
| 24 x 20“ | 465 | 455 | - | 1220 | 540 | 112 | II |
| 24 x 30“ | 465 | 455 | - | 1470 | 790 | 151 | III |
| 24 x 40“ | 465 | 455 | - | 1720 | 1040 | 190 | III |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 2.5“ (63.5mm x 1.65mm) [Standard] | 318 | 509 | 563 | 318 | 643 | 264 |
| 3.0“ (76.2mm x 1.65mm) | 292 | 578 | 252 | 292 | 660 | 252 |
| 4.0“ ( 101.6mm x 2.11mm) | 300 | 566 | 245 | 300 | 647 | 245 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN65 [Standard] | 318 | 618 | 291 | 318 | 698 | 291 |
| DN80 | 331 | 616 | 295 | 331 | 696 | 295 |
| DN100 | 300 | 638 | 281 | 300 | 718 | 281 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) [Standard] | 318 | 539 | 252 | 318 | 518 | 252 |
| 3.0“ (76.2mm x 1.65mm) | 292 | 554 | 239 | 292 | 547 | 239 |
| DN4“ (101.6mm x 2.11mm) | 300 | 534 | 229 | 300 | 603 | 229 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 24 x 10“ | 18.0 | 88.0 | 80x63x64 | 95.0 |
| 24 x 20“ | 27.0 | 97.0 | 105x63x64 | 105.0 |
| 24 x 30“ | 35.0 | 105.0 | 130x63x64 | 113.0 |
| 24 x 40“ | 44.0 | 114.0 | 155x63x64 | 122.0 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ | Standard Dimension |
|  | 24 | 30" | DIN 11851 Male Part | DN 65 | Silicone | x | x | x |
|  | 24 | 30" | DIN 11851 Male Part | DN 80 | Silicone | x | x |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 100 | Silicone | x | x |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 65 | EPDM | x | x | x |
|  | 24 | 30" | DIN 11851 Male Part | DN 80 | EPDM | x | x |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 100 | EPDM | x | x |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 65 | Silicone | x | - | x |
|  | 24 | 30" | DIN 11851 Male Part | DN 80 | Silicone | x | - |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 100 | Silicone | x | - |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 65 | EPDM | x | - | x |
|  | 24 | 30" | DIN 11851 Male Part | DN 80 | EPDM | x | - |  |
|  | 24 | 30" | DIN 11851 Male Part | DN 100 | EPDM | x | - |  |
|  | 24 | 30" | TC | 2,5" | Silicone | x | x | x |
|  | 24 | 30" | TC | 3" | Silicone | x | x |  |
|  | 24 | 30" | TC | 4" | Silicone | x | x |  |
|  | 24 | 30" | TC | 2,5" | EPDM | x | x | x |
|  | 24 | 30" | TC | 3" | EPDM | x | x |  |
|  | 24 | 30" | TC | 4" | EPDM | x | x |  |
|  | 24 | 30" | TC | 2,5" | Silicone | x | - | x |
|  | 24 | 30" | TC | 3" | Silicone | x | - |  |
|  | 24 | 30" | TC | 4" | Silicone | x | - |  |
|  | 24 | 30" | TC | 2,5" | EPDM | x | - | x |
|  | 24 | 30" | TC | 3" | EPDM | x | - |  |
|  | 24 | 30" | TC | 4" | EPDM | x | - |  |
|  | 24 | 30" | Welding Stubs | 63,5 x 1.65 | Silicone | x | x | x |
|  | 24 | 30" | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |  |
|  | 24 | 30" | Welding Stubs | 101.6 x 2.11 | Silicone | x | x |  |
|  | 24 | 30" | Welding Stubs | 63,5 x 1.65 | EPDM | x | x | x |
|  | 24 | 30" | Welding Stubs | 76.2 x 1.65 | EPDM | x | x |  |
|  | 24 | 30" | Welding Stubs | 101.6 x 2.11 | EPDM | x | x |  |
|  | 24 | 30" | Welding Stubs | 63,5 x 1.65 | Silicone | x | - | x |
|  | 24 | 30" | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |  |
|  | 24 | 30" | Welding Stubs | 101.6 x 2.11 | Silicone | x | - |  |
|  | 24 | 30" | Welding Stubs | 63,5 x 1.65 | EPDM | x | - | x |
|  | 24 | 30" | Welding Stubs | 76.2 x 1.65 | EPDM | x | - |  |
|  | 24 | 30" | Welding Stubs | 101.6 x 2.11 | EPDM | x | - |  |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ | Standard Dimension |
|  | 24 | 40" | DIN 11851 Male Part | DN 65 | Silicone | x | x | x |
|  | 24 | 40" | DIN 11851 Male Part | DN 80 | Silicone | x | x |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 100 | Silicone | x | x |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 65 | EPDM | x | x | x |
|  | 24 | 40" | DIN 11851 Male Part | DN 80 | EPDM | x | x |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 100 | EPDM | x | x |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 65 | Silicone | x | - | x |
|  | 24 | 40" | DIN 11851 Male Part | DN 80 | Silicone | x | - |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 100 | Silicone | x | - |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 65 | EPDM | x | - | x |
|  | 24 | 40" | DIN 11851 Male Part | DN 80 | EPDM | x | - |  |
|  | 24 | 40" | DIN 11851 Male Part | DN 100 | EPDM | x | - |  |
|  | 24 | 40" | TC | 2,5" | Silicone | x | x | x |
|  | 24 | 40" | TC | 3" | Silicone | x | x |  |
|  | 24 | 40" | TC | 4" | Silicone | x | x |  |
|  | 24 | 40" | TC | 2,5" | EPDM | x | x | x |
|  | 24 | 40" | TC | 3" | EPDM | x | x |  |
|  | 24 | 40" | TC | 4" | EPDM | x | x |  |
|  | 24 | 40" | TC | 2,5" | Silicone | x | - | x |
|  | 24 | 40" | TC | 3" | Silicone | x | - |  |
|  | 24 | 40" | TC | 4" | Silicone | x | - |  |
|  | 24 | 40" | TC | 2,5" | EPDM | x | - | x |
|  | 24 | 40" | TC | 3" | EPDM | x | - |  |
|  | 24 | 40" | TC | 4" | EPDM | x | - |  |
|  | 24 | 40" | Welding Stubs | 63,5 x 1.65 | Silicone | x | x | x |
|  | 24 | 40" | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |  |
|  | 24 | 40" | Welding Stubs | 101.6 x 2.11 | Silicone | x | x |  |
|  | 24 | 40" | Welding Stubs | 63,5 x 1.65 | EPDM | x | x | x |
|  | 24 | 40" | Welding Stubs | 76.2 x 1.65 | EPDM | x | x |  |
|  | 24 | 40" | Welding Stubs | 101.6 x 2.11 | EPDM | x | x |  |
|  | 24 | 40" | Welding Stubs | 63,5 x 1.65 | Silicone | x | - | x |
|  | 24 | 40" | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |  |
|  | 24 | 40" | Welding Stubs | 101.6 x 2.11 | Silicone | x | - |  |
|  | 24 | 40" | Welding Stubs | 63,5 x 1.65 | EPDM | x | - | x |
|  | 24 | 40" | Welding Stubs | 76.2 x 1.65 | EPDM | x | - |  |
|  | 24 | 40" | Welding Stubs | 101.6 x 2.11 | EPDM | x | - |  |

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=443.40 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 4 |
| 6 | FOOT | AISI 304 |  | 4 |
| 7 | CLAMP 4“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 4“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=136.50 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

Fluid group 1 (dangerous) on request

article no., price u delivery time can be different.

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | R mm | S mm | N mm | C mm | E mm | VOLUME liters (V) | PED Category |
| 30 x 10“ | 550 | 550 | - | 1000 | 290 | 118 | III |
| 30 x 20“ | 550 | 550 | - | 1250 | 540 | 176 | III |
| 30 x 30“ | 550 | 550 | - | 1500 | 790 | 235 | III |
| 30 x 40“ | 550 | 550 | - | 1750 | 1040 | 293 | III |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| 3.0“ (76.2mm x 1.65mm) [Standard] | 292 | 685 | 323 | 292 | 765 | 323 |
| 4.0“ ( 101.6mm x 2.11mm) | 300 | 662 | 306 | 30 | 742 | 306 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | Q | A | L | Q |
| DN80 [Standard] | 331 | 712 | 356 | 331 | 792 | 356 |
| DN100 | 300 | 734 | 342 | 300 | 814 | 342 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 3.0“ (76.2mm x 1.65mm) [Standard] | 292 | 660 | 310 | 292 | 740 | 310 |
| DN 4.0“ (101.6mm x 2.11mm) | 300 | 630 | 290 | 300 | 710 | 290 |

| PACKAGING SIZE AND WEIGHT |  |  |  |  |
| --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | PACKAGING SIZE [cm] | TOTAL WEIGHT PACKED [kg] |
| 30 x 10“ | 28.0 | 93.0 | 80x100x78 | 112.0 |
| 30 x 20“ | 38.0 | 101.0 | 105x100x78 | 121.0 |
| 30 x 30“ | 48.0 | 109.0 | 130x100x78 | 131.0 |
| 30 x 40“ | 58.0 | 117.0 | 155x100x78 | 140.0 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ | Standard Dimension |
|  | 30 | 30“ | DIN 11851 Male Part | DN 80 | Silicone | x | x | x |
|  | 30 | 30“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |  |
|  | 30 | 30“ | DIN 11851 Male Part | DN 80 | EPDM | x | x | x |
|  | 30 | 30“ | DIN 11851 Male Part | DN 100 | EPDM | x | x |  |
|  | 30 | 30“ | DIN 11851 Male Part | DN 80 | Silicone | x | - | x |
|  | 30 | 30“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |  |
|  | 30 | 30“ | DIN 11851 Male Part | DN 80 | EPDM | x | - | x |
|  | 30 | 30“ | DIN 11851 Male Part | DN 100 | EPDM | x | - |  |
|  | 30 | 30“ | TC | 3.0“ | Silicone | x | x | x |
|  | 30 | 30“ | TC | 4.0“ | Silicone | x | x |  |
|  | 30 | 30“ | TC | 3.0“ | EPDM | x | x | x |
|  | 30 | 30“ | TC | 4.0“ | EPDM | x | x |  |
|  | 30 | 30“ | TC | 3.0“ | Silicone | x | - | x |
|  | 30 | 30“ | TC | 4.0“ | Silicone | x | - |  |
|  | 30 | 30“ | TC | 3.0“ | EPDM | x | - | x |
|  | 30 | 30“ | TC | 4.0“ | EPDM | x | - |  |
|  | 30 | 30“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x | x |
|  | 30 | 30“ | Welding Stubs | 101.6 x 1.65 | Silicone | x | x |  |
|  | 30 | 30“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | x | x |
|  | 30 | 30“ | Welding Stubs | 101.6 x 1.65 | EPDM | x | x |  |
|  | 30 | 30“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - | x |
|  | 30 | 30“ | Welding Stubs | 101.6 x 1.65 | Silicone | x | - |  |
|  | 30 | 30“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | - | x |
|  | 30 | 30“ | Welding Stubs | 101.6 x 1.65 | EPDM | x | - |  |
|  | 30 | 40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x | x |
|  | 30 | 40“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |  |
|  | 30 | 40“ | DIN 11851 Male Part | DN 80 | EPDM | x | x | x |
|  | 30 | 40“ | DIN 11851 Male Part | DN 100 | EPDM | x | x |  |
|  | 30 | 40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - | x |
|  | 30 | 40“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |  |
|  | 30 | 40“ | DIN 11851 Male Part | DN 80 | EPDM | x | - | x |
|  | 30 | 40“ | DIN 11851 Male Part | DN 100 | EPDM | x | - |  |
|  | 30 | 40“ | TC | 3.0“ | Silicone | x | x | x |
|  | 30 | 40“ | TC | 4.0“ | Silicone | x | x |  |
|  | 30 | 40“ | TC | 3.0“ | EPDM | x | x | x |
|  | 30 | 40“ | TC | 4.0“ | EPDM | x | x |  |
|  | 30 | 40“ | TC | 3.0“ | Silicone | x | - | x |
|  | 30 | 40“ | TC | 4.0“ | Silicone | x | - |  |
|  | 30 | 40“ | TC | 3.0“ | EPDM | x | - | x |
|  | 30 | 40“ | TC | 4.0“ | EPDM | x | - |  |
|  | 30 | 40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x | x |
|  | 30 | 40“ | Welding Stubs | 101.6 x 1.65 | Silicone | x | x |  |
|  | 30 | 40“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | x | x |
|  | 30 | 40“ | Welding Stubs | 101.6 x 1.65 | EPDM | x | x |  |
|  | 30 | 40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - | x |
|  | 30 | 40“ | Welding Stubs | 101.6 x 1.65 | Silicone | x | - |  |
|  | 30 | 40“ | Welding Stubs | 76.2 x 1.65 | EPDM | x | - | x |
|  | 30 | 40“ | Welding Stubs | 101.6 x 1.65 | EPDM | x | - |  |

XX = 25 (Adapter 25 / code 7)	37

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=532.30 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 4 |
| 6 | FOOT | AISI 304 |  | 4 |
| 7 | CLAMP 4“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 4“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=164.46 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

Fluid group 1 (dangerous) on request

| TECHNICHAL DATA |  |
| --- | --- |
| VESSEL RANGE | PRM |
| NO. CARTRIDGED x HEIGHT | 36 x 30“ |
| CARTRIDGE SOCKET | 226 CODE 7 |
| ATEX REGULATION | GROUP 2 - Cat 2 GASuDUST FLUID GR IIC zone 1u21 |
| DESIGN CODE | DIN EN 13445 |
| CERTIFICATION | PED 2014/68/EU |
| PED CATEGORY | III |
| DESIGN PRESSURE -20°C +40°C (PS) | -1bar...+10bar |
| DESIGN PRESSURE -20°C +150°C (PS) | -1bar...+4bar |
| DESIGN TEMP. MIN-MAX (TS) | -20°C...+150°C |
| VESSEL MATERIAL | 316L / 1.4404 |
| TEST PRESSURE (PT) | 20 bar |
| TOTAL VESSEL VOLUME liters (V) | 235 |
| FLUIDGROUP | 2 |
| PHYSICAL STATE | LIQUID / GAS |
| SEALS MATERIAL | SILICONE |
| INTERNAL FINISH | MECHANICAL POLISHING |
| INTERNAL ROUGHNESS | RA <0.8 MICRON |
| EXTERNAL FINISH | MECHANICAL POLISHING |
| VENT CONNECTION A | CLAMP ASME 1.5“ |
| IN/OUT CONNECTION B | DIN11851-DN100 MALE PART |
| DRAIN CONNECTION C | CLAMP ASME 1.5“ |

article no., price u delivery time can be differ.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ | Standard Diameter |
|  | 36 | 30“ | DIN 11851 Male Part | DN 100 | Silicone | x | - | x |
|  | 36 | 30“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |  |
|  | 36 | 30“ | DIN 11851 Male Part | DN 100 | EPDM | x | - | x |
|  | 36 | 30“ | DIN 11851 Male Part | DN 100 | EPDM | x | x |  |

XX = 25 (Adapter 25 / code 7)	40

XX = 28 (Sartorius Adapter 28 tripple Bayonet)

# Type 810

## Fluid group 1 (dangerous) Series SH

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 01 x 10“ | 790 | 300 | 3.0 | I | 1 (dangerous) |
| 01 x 20“ | 1040 | 550 | 4.8 | I | 1 (dangerous) |
| 01 x 30“ | 1290 | 800 | 6.7 | II | 1 (dangerous) |
| 01 x 40“ | 1540 | 1050 | 8.6 | II | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.0“ (25.4mm x 1.65mm) | 375 | 189 | 104 | 375 | 270 | 154 |
| 1.5“ (38,1mm x 1.65mm) | 355 | 220 | 122 | 355 | 370 | 198 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN25 (25.4mm x 1.65mm) | 375 | 221 | 120 | 375 | 302 | 170 |
| DN40 (38,1mm x 1.65mm) | 355 | 258 | 144 | 355 | 415 | 217 |
| BUTT WELD ASME BPE |  |  |  |  |  |  |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 1.0“ (25.4mm x 1.65mm) | 375 | 163 | 91 | 375 | 244 | 141 |
| 1.5“ (38.1mm x 1.65mm) | 355 | 194 | 112 | 355 | 345 | 185 |

| SMS MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.0“ (25.4mm x 1.65mm) | 375 | 203 | 111 | 375 | 284 | 161 |
| 1.5“ (38,1mm x 1.65mm) | 355 | 240 | 133 | 355 | 391 | 208 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.0“ |
|  | 01 | 10“-40“ | DIN 11851 Male Part | DN 25 | Silicone | x | x |
|  | 01 | 10“-40“ | DIN 11851 Male Part | DN 25 | Silicone | x | - |
|  | 01 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 01 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 01 | 10“-40“ | TC | 1“ | Silicone | x | x |
|  | 01 | 10“-40“ | TC | 1.5“ | Silicone | x | - |
|  | 01 | 10“-40“ | TC | 1“ | Silicone | x | x |
|  | 01 | 10“-40“ | TC | 1.5“ | Silicone | x | - |
|  | 01 | 10“-40“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 10“-40“ | Welding Stubs | 381. x 1.65 | Silicone | x | - |
|  | 01 | 10“-40“ | Welding Stubs | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 01 | 10“-40“ | SMS - Male part | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 10“-40“ | SMS - Male part | 381. x 1.65 | Silicone | x | - |
|  | 01 | 10“-40“ | SMS - Male part | 25.4 x 1.65 | Silicone | x | x |
|  | 01 | 10“-40“ | SMS - Male part | 38.1 x 1.65 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | CLAMP 4“ | AISI 304 |  | 1 |
| 3 | CARFTRIDGES PLATE + BENDS | AISI 316L |  | 1 |
| 4 5 | GASKET CLAMP 4“ | SILICONE |  | 1 |
|  | LEG | AISI 304 |  | 3 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 03 x 10“ | 816 | 290 | 7.3 | II | 1 (dangerous) |
| 03 x 20“ | 1065 | 540 | 12.6 | II | 1 (dangerous) |
| 03 x 30“ | 1314 | 790 | 17.8 | II | 1 (dangerous) |
| 03 x 40“ | 1564 | 1040 | 23.1 | III | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 310 | 148 | 345 | 385 | 223 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN40 (38.1mm x 1.65mm) | 345 | 351 | 168 | 345 | 426 | 243 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 1.5“ (38.1mm x 1.65mm) | 345 | 285 | 135 | 345 | 360 | 210 |

| SMS MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38,1mm x 1.65mm) | 345 | 331 | 158 | 345 | 406 | 233 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 03 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 03 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 03 | 10“-40“ | TC | 1.5“ | Silicone | x | x |
|  | 03 | 10“-40“ | TC | 1.5“ | Silicone | x | - |
|  | 03 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 03 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 03 | 10“-40“ | SMS - Male part | 38.1 x 1.65 | Silicone | x | x |
|  | 03 | 10“-40“ | SMS - Male part | 38.1 x 1.65 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=158.11 | SILICONE |  | 1 |
| 5 | GASKET CLAMP 1.5“ | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L |  | 1 |
| 8 | OUTLET BEND | AISI 316L |  | 1 |
| 9 | COMPLETE LEG | AISI 304 |  | 3 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 05 x 10“ | 825 | 290 | 11 | II | 1 (dangerous) |
| 05 x 20“ | 1075 | 540 | 20 | II | 1 (dangerous) |
| 05 x 30“ | 1325 | 790 | 29.1 | III | 1 (dangerous) |
| 05 x 40“ | 1575 | 1040 | 38.2 | III | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 331 | 168 | 345 | 406 | 243 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 379 | 196 | 315 | 446 | 263 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN40 (38.1mm x 1.65mm) | 345 | 371 | 188 | 345 | 446 | 263 |
| DN50 (50.8mm x 1.65mm) | 315 | 423 | 218 | 315 | 490 | 285 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 1.5“ (38.1mm x 1.65mm) | 345 | 305 | 155 | 345 | 380 | 230 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 353 | 183 | 315 | 420 | 250 |

| SMS MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38,1mm x 1.65mm) | 345 | 351 | 178 | 345 | 426 | 253 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 403 | 208 | 315 | 470 | 275 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 05 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 05 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 05 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 05 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 05 | 10“-40“ | TC | 1.5“ | Silicone | x | x |
|  | 05 | 10“-40“ | TC | 1.5“ | Silicone | x | - |
|  | 05 | 10“-40“ | TC | 2“ | Silicone | x | x |
|  | 05 | 10“-40“ | TC | 2“ | Silicone | x | - |
|  | 05 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 05 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 05 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 05 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 05 | 10“-40“ | SMS - Male part | 381. x 1.65 | Silicone | x | x |
|  | 05 | 10“-40“ | SMS - Male part | 381. x 1.65 | Silicone | x | - |
|  | 05 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | x |
|  | 05 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=208.91 | SILICONE |  | 1 |
| 5 | GASKET CLAMP 2“ | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L |  | 1 |
| 8 | OUTLET BEND | AISI 316L |  | 1 |
| 9 | COMPLETE LEG | AISI 304 |  | 3 |
| 11 | PROTECTION PLATE | AISI 316L |  | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 08 x 10“ | 846 | 290 | 21.3 | III | 1 (dangerous) |
| 08 x 20“ | 1096 | 540 | 35.5 | III | 1 (dangerous) |
| 08 x 30“ | 1346 | 790 | 49.7 | III | 1 (dangerous) |
| 08 x 40“ | 1596 | 1040 | 63.9 | III | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 357 | 194 | 345 | 432 | 269 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 405 | 222 | 315 | 472 | 289 |
| 2.5“ (63.5mm x 1.65mm) | 286 | 468 | 243 | 286 | 548 | 323 |
| 3.0“ (76.2mm x 1.65mm) | 258 | 524 | 271 | 258 | 604 | 351 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN40 (38.1mm x 1.65mm) | 345 | 397 | 214 | 345 | 472 | 289 |
| DN50 (50.8mm x 1.65mm) | 315 | 449 | 244 | 315 | 516 | 311 |
| DN65 (63.5mm x 1.65mm) | 286 | 518 | 268 | 286 | 602 | 350 |
| DN80 (76.2mm x 1.65mm) | 258 | 588 | 303 | 258 | 658 | 383 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 1.5“ (38.1mm x 1.65mm) | 345 | 331 | 181 | 345 | 406 | 256 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 379 | 209 | 315 | 446 | 276 |
| 2.5“ (63.5mm x 1.65mm) | 286 | 442 | 230 | 286 | 522 | 310 |
| 3.0“ (76.2mm x 1.65mm) | 258 | 498 | 258 | 258 | 578 | 338 |

| SMS - Male part |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 1.5“ (38.1mm x 1.65mm) | 345 | 377 | 204 | 345 | 452 | 279 |
| 2.0“ (50.8mm x 1.65mm) | 315 | 429 | 234 | 315 | 496 | 301 |
| 2.5“ (63.5mm x 1.65mm) | 286 | 502 | 260 | 286 | 582 | 340 |
| 3.0“ (76.2mm x 1.65mm) | 258 | 568 | 293 | 258 | 648 | 373 |

### Please Note

In filter housings with a flange connection at the in- and outlet e.g. DIN-EN1092-1, ANSI, etc. the bends will be welded directly to the baseplate, instead of the standard flange connection described in the detailed drawing.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | x |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 40 | Silicone | x | - |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |
|  | 08 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |
|  | 08 | 10“-40“ | TC | 1.5“ | Silicone | x | x |
|  | 08 | 10“-40“ | TC | 1.5“ | Silicone | x | - |
|  | 08 | 10“-40“ | TC | 2“ | Silicone | x | x |
|  | 08 | 10“-40“ | TC | 2“ | Silicone | x | - |
|  | 08 | 10“-40“ | TC | 2.5“ | Silicone | x | x |
|  | 08 | 10“-40“ | TC | 2.5“ | Silicone | x | - |
|  | 08 | 10“-40“ | TC | 3.0“ | Silicone | x | x |
|  | 08 | 10“-40“ | TC | 3.0“ | Silicone | x | - |
|  | 08 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | Welding Stubs | 38.1 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | SMS - Male part | 38.1 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | SMS - Male part | 38.1 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | - |
|  | 08 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | x |
|  | 08 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | V-BAND | AISI 304 |  | 1 |
| 3 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 4 | O-RING d=5.35 ID=266.06 | SILICONE |  | 1 |
| 5 | GASKET CLAMP | PTFE |  | 2 |
| 6 | BEND FLANGE | AISI 304 |  | 4 |
| 7 | INLET BEND | AISI 316L | - | 1 |
| 8 | OUTLET BEND | AISI 316L | - | 1 |
| 9 | COMPLETE LEG | AISI 304 | - | 3 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 12 x 10“ | 930 | 290 | 41 | III | 1 (dangerous) |
| 12 x 20“ | 1180 | 540 | 66 | III | 1 (dangerous) |
| 12 x 30“ | 1430 | 790 | 89 | III | 1 (dangerous) |
| 12 x 40“ | 1680 | 1040 | 113 | IV | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.0“ (50.8mm x 1.65mm) | 350 | 474 | 238 | 350 | 544 | 238 |
| 2.5“ (63.5mm x 1.65mm) | 326 | 493 | 225 | 326 | 573 | 225 |
| 3.0“ (76.2mm x 1.65mm) | 298 | 549 | 253 | 298 | 629 | 253 |
| 4.0“ (101.6mm x 2mm) | 290 | 529 | 243 | 290 | 609 | 243 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN50 (50.8mm x 1.65mm) | 350 | 519 | 258 | 350 | 586 | 258 |
| DN65 (63.5mm x 1.65mm) | 326 | 547 | 252 | 326 | 627 | 252 |
| DN80 (76.2mm x 1.65mm) | 297 | 603 | 280 | 297 | 683 | 280 |
| DN100 (101.6mm x 2mm) | 290 | 527 | 202 | 290 | 687 | 282 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 2.0“ (50.8mm x 1.65mm) | 350 | 449 | 225 | 350 | 518 | 225 |
| 2.5“ (63.5mm x 1.65mm) | 326 | 467 | 212 | 326 | 547 | 212 |
| 3.0“ (76.2mm x 1.65mm) | 298 | 523 | 240 | 298 | 603 | 240 |
| 4.0“ (101.6mm x 2mm) | 290 | 483 | 210 | 290 | 583 | 230 |

| SMS - Male part |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.0“ (50.8mm x 1.65mm) | 350 | 499 | 250 | 350 | 568 | 250 |
| 2.5“ (63.5mm x 1.65mm) | 326 | 527 | 242 | 326 | 607 | 242 |
| 3.0“ (76.2mm x 1.65mm) | 298 | 593 | 275 | 298 | 673 | 275 |
| 4.0“ (101.6mm x 2mm) | 290 | 543 | 240 | 290 | 643 | 260 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | x |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 50 | Silicone | x | - |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |
|  | 12 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |
|  | 12 | 10“-40“ | TC | 2.0“ | Silicone | x | x |
|  | 12 | 10“-40“ | TC | 2.0“ | Silicone | x | - |
|  | 12 | 10“-40“ | TC | 2.5“ | Silicone | x | x |
|  | 12 | 10“-40“ | TC | 2.5“ | Silicone | x | - |
|  | 12 | 10“-40“ | TC | 3.0“ | Silicone | x | x |
|  | 12 | 10“-40“ | TC | 3.0“ | Silicone | x | - |
|  | 12 | 10“-40“ | TC | 4.0“ | Silicone | x | x |
|  | 12 | 10“-40“ | TC | 4.0“ | Silicone | x | - |
|  | 12 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | Welding Stubs | 50.8 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | x |
|  | 12 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | - |
|  | 12 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | SMS - Male part | 50.8 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | x |
|  | 12 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | - |
|  | 12 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | x |
|  | 12 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L | - | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=342.30 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 3 |
| 6 | FOOT | AISI 304 |  | 3 |
| 7 | CLAMP 3“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 3“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=100.00 | SILICONE | - | 2 |
| 9 | INLET BEND | AISI 316L | - | 1 |
| 10 | OUTLET BEND | AISI 316L | - | 1 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 18 x 10“ | 950 | 290 | 54 | III | 1 (dangerous) |
| 18 x 20“ | 1200 | 540 | 85 | III | 1 (dangerous) |
| 18 x 30“ | 1450 | 790 | 115 | III | 1 (dangerous) |
| 18 x 40“ | 1700 | 1040 | 146 | IV | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 325 | 509 | 225 | 325 | 589 | 225 |
| 3.0“ (76.2mm x 1.65mm) | 297 | 566 | 253 | 297 | 646 | 253 |
| 4.0“ (101.6mm x 2mm) | 287 | 552 | 246 | 287 | 632 | 246 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN65 | 325 | 564 | 252 | 325 | 644 | 252 |
| DN80 | 296 | 620 | 280 | 296 | 700 | 280 |
| DN100 | 287 | 604 | 262 | 287 | 704 | 282 |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 2.5“ (63.5mm x 1.65mm) | 325 | 484 | 212 | 325 | 564 | 212 |
| 3.0“ (76.2mm x 1.65mm) | 297 | 540 | 240 | 297 | 620 | 240 |
| 4.0“ (101.6mm x 2mm) | 292 | 604 | 314 | 292 | 600 | 230 |

| SMS - Male part |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 325 | 544 | 242 | 325 | 624 | 242 |
| 3.0“ (76.2mm x 1.65mm) | 297 | 610 | 275 | 297 | 690 | 275 |
| 4.0“ (101.6mm x 2mm) | 287 | 580 | 260 | 287 | 660 | 260 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |
|  | 18 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |
|  | 18 | 10“-40“ | TC | 2.5“ | Silicone | x | x |
|  | 18 | 10“-40“ | TC | 2.5“ | Silicone | x | - |
|  | 18 | 10“-40“ | TC | 3.0“ | Silicone | x | x |
|  | 18 | 10“-40“ | TC | 3.0“ | Silicone | x | - |
|  | 18 | 10“-40“ | TC | 4.0“ | Silicone | x | x |
|  | 18 | 10“-40“ | TC | 4.0“ | Silicone | x | - |
|  | 18 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 18 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 18 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |
|  | 18 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |
|  | 18 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | x |
|  | 18 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | - |
|  | 18 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | x |
|  | 18 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | - |
|  | 18 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | x |
|  | 18 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | - |
|  | 18 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | x |
|  | 18 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=393.10 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 3 |
| 6 | FOOT | AISI 304 |  | 3 |
| 7 | CLAMP 3“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 3“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=110.50 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L | - | 1 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 24 x 10“ | 970 | 290 | 73 | III | 1 (dangerous) |
| 24 x 20“ | 1220 | 540 | 112 | III | 1 (dangerous) |
| 24 x 30“ | 1470 | 790 | 151 | IV | 1 (dangerous) |
| 24 x 40“ | 1720 | 1040 | 190 | IV | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 318 | 564 | 265 | 318 | 644 | 265 |
| 3.0“ (76.2mm x 1.65mm) | 292 | 580 | 253 | 292 | 660 | 253 |
| 4.0“ ( 101.6mm x 2.11mm) | 300 | 566 | 245 | 300 | 647 | 246 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN65 (63.5mm x 1.65mm) | 318 | 618 | 292 | 318 | 698 | 292 |
| DN80 (85mm x 2mm) | 331 | 616 | 296 | 331 | 696 | 296 |
| DN100 (101.6mm x 2mm) | 300 | 638 | 281 | 300 | 718 | 296 |
| DN125 (129.0mm x 2mm) | 253 | 701 | 314 | - | - | - |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 2.5“ (63.5mm x 1.65mm) | 318 | 538 | 252 | 318 | 618 | 252 |
| 3.0“ (76.2mm x 1.65mm) | 292 | 554 | 240 | 292 | 634 | 240 |
| 4.0“ (101.6mm x 2mm) | 300 | 534 | 229 | 300 | 614 | 230 |
| 5.0“ (139.7mm x 2.60mm) | 248 | 615 | 271 | - | - | - |

| SMS - Male part |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 318 | 598 | 282 | 318 | 678 | 282 |
| 3.0“ (76.2mm x 1.65mm) | 292 | 624 | 275 | 292 | 704 | 275 |
| 4.0“ (101.6mm x 2mm) | 300 | 594 | 259 | 300 | 674 | 259 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Dia- meter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |
|  | 24 | 10“-40“ | DIN 11851 Male Part | DN 125 | Silicone | x | - |
|  | 24 | 10“-40“ | TC | 2.5“ | Silicone | x | x |
|  | 24 | 10“-40“ | TC | 2.5“ | Silicone | x | - |
|  | 24 | 10“-40“ | TC | 3.0“ | Silicone | x | x |
|  | 24 | 10“-40“ | TC | 3.0“ | Silicone | x | - |
|  | 24 | 10“-40“ | TC | 4.0“ | Silicone | x | x |
|  | 24 | 10“-40“ | TC | 4.0“ | Silicone | x | - |
|  | 24 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 24 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 24 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |
|  | 24 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |
|  | 24 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | x |
|  | 24 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | - |
|  | 24 | 10“-40“ | Welding Stubs | 139.7 x 2.60 | Silicone | x | - |
|  | 24 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | x |
|  | 24 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | - |
|  | 24 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | x |
|  | 24 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | - |
|  | 24 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | x |
|  | 24 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=443.40 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 4 |
| 6 | FOOT | AISI 304 |  | 4 |
| 7 | CLAMP 4“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 4“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=136.50 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C mm | E mm | VOLUME liters (V) | PED Category | Fluid Group |
| 30 x 10“ | 1000 | 290 | 118 | III | 1 (dangerous) |
| 30 x 20“ | 1250 | 540 | 176 | III | 1 (dangerous) |
| 30 x 30“ | 1500 | 790 | 235 | III | 1 (dangerous) |
| 30 x 40“ | 1750 | 1040 | 293 | III | 1 (dangerous) |

| CLAMP |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 318 | 665 | 330 | 318 | 745 | 330 |
| 3.0“ (76.2mm x 1.65mm) | 291 | 686 | 323 | 291 | 766 | 323 |
| 4.0“ ( 101.6mm x 2.11mm) | 300 | 662 | 306 | 300 | 742 | 306 |

| DIN 11851 MALE PART |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| DN65 (63.5mm x 1.65mm) | 318 | 719 | 357 | 318 | 800 | 357 |
| DN80 (85mm x 2mm) | 331 | 712 | 356 | 331 | 792 | 357 |
| DN100 (101.6mm x 2mm) | 300 | 734 | 342 | 300 | 814 | 342 |
| DN125 (129.0mm x 2mm) | 254 | 785 | 362 | - | - | - |
| DN150 (154.0mm x 2mm) | 218 | 820 | 355 | - | - | - |

| BUTT WELD ASME BPE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | L | P | A | L | P |
| 2.5“ (63.5mm x 1.65mm) | 318 | 639 | 317 | 318 | 719 | 317 |
| 3.0“ (76.2mm x 1.65mm) | 291 | 660 | 310 | 291 | 740 | 310 |
| 4.0“ (101.6mm x 2mm) | 300 | 630 | 290 | 300 | 710 | 290 |
| 5.0“ (139.7mm x 2.60mm) | 251 | 671 | 291 | - | - | - |
| 6.0“ (168.3mm x 2.60mm) | 210 | 747 | 329 | - | - | - |

| SMS - Male part |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WITHOUT DRAIN |  |  | WITH DRAIN |  |  |
| Dimension [mm] | A | B | Q | A | B | Q |
| 2.5“ (63.5mm x 1.65mm) | 318 | 699 | 347 | 318 | 779 | 347 |
| 3.0“ (76.2mm x 1.65mm) | 291 | 730 | 345 | 291 | 810 | 345 |
| 4.0“ (101.6mm x 2mm) | 300 | 690 | 320 | 300 | 770 | 320 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Dia- meter | Gasket | With Vent TC 1.5“ | With Drain TC 1.5“ |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | x |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 65 | Silicone | x | - |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | x |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 80 | Silicone | x | - |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | x |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 100 | Silicone | x | - |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 125 | Silicone | x | - |
|  | 30 | 10“-40“ | DIN 11851 Male Part | DN 150 | Silicone | x | - |
|  | 30 | 10“-40“ | TC | 2.5“ | Silicone | x | x |
|  | 30 | 10“-40“ | TC | 2.5“ | Silicone | x | - |
|  | 30 | 10“-40“ | TC | 3.0“ | Silicone | x | x |
|  | 30 | 10“-40“ | TC | 3.0“ | Silicone | x | - |
|  | 30 | 10“-40“ | TC | 4.0“ | Silicone | x | x |
|  | 30 | 10“-40“ | TC | 4.0“ | Silicone | x | - |
|  | 30 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | x |
|  | 30 | 10“-40“ | Welding Stubs | 63.5 x 1.65 | Silicone | x | - |
|  | 30 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | x |
|  | 30 | 10“-40“ | Welding Stubs | 76.2 x 1.65 | Silicone | x | - |
|  | 30 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | x |
|  | 30 | 10“-40“ | Welding Stubs | 101.6 x 2.00 | Silicone | x | - |
|  | 30 | 10“-40“ | Welding Stubs | 139.7 x 2.60 | Silicone | x | - |
|  | 30 | 10“-40“ | Welding Stubs | 168.3 x 2.60 | Silicone | x | - |
|  | 30 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | x |
|  | 30 | 10“-40“ | SMS - Male part | 63.5 x 1.65 | Silicone | x | - |
|  | 30 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | x |
|  | 30 | 10“-40“ | SMS - Male part | 76.2 x 1.65 | Silicone | x | - |
|  | 30 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | x |
|  | 30 | 10“-40“ | SMS - Male part | 101.6 x 2.00 | Silicone | x | - |

„X“ = Placeholder for the bowl height:

1 = 10“

2 = 20“

3 = 30“

4 = 40“

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | BOLT CLAMP M16 | AISI 316Ti |  |  |
| 3 | O-RING d=7.00 ID=532.30 | SILICONE |  | 1 |
| 4 | CARTRIDGE PLATE | AISI 316L |  | 1 |
| 5 | LEG | AISI 304 |  | 4 |
| 6 | FOOT | AISI 304 |  | 4 |
| 7 | CLAMP 4“ | AISI 304 |  | 2 |
| 8.1 | GASKET CLAMP 4“ | PTFE |  | 2 |
| 8.2 | O-RING d=5.34 ID=164.46 | SILICONE |  | 2 |
| 9 | INLET BEND | AISI 316L |  | 1 |
| 10 | OUTLET BEND | AISI 316L |  | 1 |
| 11 | PROTECTION PLATE | AISI 316L | - | 1 |

| TECHNICHAL DATA |  |
| --- | --- |
| NO. CARTRIDGED | 01 |
| CARTRIDGE SOCKET | 226 CODE 7 |
| ATEX REGULATION | GROUP 2 - Cat 2 GASuDUST FLUID GR IIC zone 1u21 |
| DESIGN CODE | DIN EN 13445 |
| CERTIFICATION | PED 2014/68/EU |
| PED CATEGORY | 4.3 |
| DESIGN PRESSURE | +10bar |
| DESIGN TEMPERATURE | -20°C...+150°C |
| VESSEL MATERIAL | 316L / 1.4404 |
| FLUIDGROUP | 2 (not dangerous) |
| PHYSICAL STATE | LIQUID / GAS |
| INTERNAL FINISH | MECHANICAL POLISHING |
| INTERNAL ROUGHNESS | RA <0.8 MICRON |
| EXTERNAL FINISH | MECHANICAL POLISHING |
| INLET CONNECTION A | CLAMP ASME 1.5“ (38.1mm x 1.65xx) |
| OUTLET CONNECTION B | CLAMP ASME 1.5“ (38.1mm x 1.65xx) |
| DRAIN CONNECTION C | CLAMP ASME 0.5“ (12.7mm x 1.65xx) |

| DIMENSIONAL DATA |  |  |  |
| --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | A mm | B mm | PED Category |
| 01 x 05“ | 272 | 227 | 4.3 |
| 01 x 10“ | 375 | 333 | 4.3 |
| 01 x 20“ | 625 | 581 | 4.3 |
| 01 x 30“ | 875 | 831 | 4.3 |

| SPARE PARTS |  |  |  |
| --- | --- | --- | --- |
| DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| CLAMP 4“ | AISI 304 |  | 1 |
| GASKET 4“ | SILICONE |  | 1 |
| GASKET 4“ | EPDM |  | 1 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection Inlet u Outlet | Nominal Diameter | Gasket | With Drain TC 0.5“ |
|  | 01 | 05“ | TC | 1.5“ | Silicone | x |
|  | 01 | 10“ | TC | 1.5“ | Silicone | x |
|  | 01 | 20“ | TC | 1.5“ | Silicone | x |
|  | 01 | 30“ | TC | 1.5“ | Silicone | x |
|  | 01 | 05“ | TC | 1.5“ | EPDM | x |
|  | 01 | 10“ | TC | 1.5“ | EPDM | x |
|  | 01 | 20“ | TC | 1.5“ | EPDM | x |
|  | 01 | 30“ | TC | 1.5“ | EPDM | x |

# Type 810 - Gas 1-round Housings

(10 bar Version)

N3

| DIMENSIONAL DATA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | C [mm] with Vent u Drain BSPF 1/4“ female | C [mm] with Vent TC 1.5“ u Drain TC 0.5“ | B [mm] with Drain BSPF 1/4“ female | B [mm] with Drain TC 0.5“ | PED Category |
| 01 x 5“ | 381 | 381 | 178 | 168 | 4.3 |
| 01 x 10“ | 501 | 501 | 178 | 168 | 4.3 |
| 01 x 20“ | 751 | 751 | 178 | 168 | 4.3 |
| 01 x 30“ | 1001 | 1001 | 178 | 168 | I |
| 01 x 40“ | 1251 | 1251 | 178 | 168 | I |

| CLAMP |  |  |  |
| --- | --- | --- | --- |
| Dimension | L [mm] | Q [mm] | N [mm] |
| 1.5“ (38.1mm x 1.65mm) | 129 | 62 | 99 |

| DIN 11851 MALE PART |  |  |  |
| --- | --- | --- | --- |
| Dimension | L [mm] | Q [mm] | N [mm] |
| DN40 | 188 | 81 | 99 |

| BUTT WELD ASME BPE |  |  |  |
| --- | --- | --- | --- |
| Dimension | L [mm] | Q [mm] | N [mm] |
| 1.5“ (38.1mm x 1.65mm) | 264 | 129 | 99 |

| PACKAGING SIZE AND WEIGHT |  |  |  |
| --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | BOWL WEIGHT [kg] | TOTAL NET WEIGHT [kg] | TOTAL WEIGHT PACKED [kg] |
| 01 x 5“ | 0.7 | 2.6 | 3.6 |
| 01 x 10“ | 1.1 | 3.0 | 4.0 |
| 01 x 20“ | 1.9 | 4.0 | 5.0 |
| 01 x 30“ | 2.7 | 5.0 | 6.0 |
| 01 x 40“ | 3.5 | 6.0 | 7.0 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection | Nominal Diameter | Gasket | With Vent u Drain BSPF 1/4“ female [G/H] | With Vent TC 1.5“ u Drain TC 0.5“ [G/H] |
|  | 01 | 5“ | TC | 1.5“ | Silicone | x |  |
|  | 01 | 5“ | TC | 1.5“ | EPDM | x |  |
|  | 01 | 5“ | Welding Stubs | 1.5“ | Silicone | x |  |
|  | 01 | 5“ | Welding Stubs | 1.5“ | EPDM | x |  |
|  | 01 | 5“ | DIN 11851 Male Part | DN 40 | Silicone | x |  |
|  | 01 | 5“ | DIN 11851 Male Part | DN 40 | EPDM | x |  |
|  | 01 | 10“ | TC | 1.5“ | Silicone | x |  |
|  | 01 | 10“ | TC | 1.5“ | EPDM | x |  |
|  | 01 | 10“ | Welding Stubs | 1.5“ | Silicone | x |  |
|  | 01 | 10“ | Welding Stubs | 1.5“ | EPDM | x |  |
|  | 01 | 10“ | DIN 11851 Male Part | DN 40 | Silicone | x |  |
|  | 01 | 10“ | DIN 11851 Male Part | DN 40 | EPDM | x |  |
|  | 01 | 20“ | TC | 1.5“ | Silicone | x |  |
|  | 01 | 20“ | TC | 1.5“ | EPDM | x |  |
|  | 01 | 20“ | Welding Stubs | 1.5“ | Silicone | x |  |
|  | 01 | 20“ | Welding Stubs | 1.5“ | EPDM | x |  |
|  | 01 | 20“ | DIN 11851 Male Part | DN 40 | Silicone | x |  |
|  | 01 | 20“ | DIN 11851 Male Part | DN 40 | EPDM | x |  |
|  | 01 | 30“ | TC | 1.5“ | Silicone | x |  |
|  | 01 | 30“ | TC | 1.5“ | EPDM | x |  |
|  | 01 | 30“ | Welding Stubs | 1.5“ | Silicone | x |  |
|  | 01 | 30“ | Welding Stubs | 1.5“ | EPDM | x |  |
|  | 01 | 30“ | DIN 11851 Male Part | DN 40 | Silicone | x |  |
|  | 01 | 30“ | DIN 11851 Male Part | DN 40 | EPDM | x |  |
|  | 01 | 40“ | TC | 1.5“ | Silicone | x |  |
|  | 01 | 40“ | TC | 1.5“ | EPDM | x |  |
|  | 01 | 40“ | Welding Stubs | 1.5“ | Silicone | x |  |
|  | 01 | 40“ | Welding Stubs | 1.5“ | EPDM | x |  |
|  | 01 | 40“ | DIN 11851 Male Part | DN 40 | Silicone | x |  |
|  | 01 | 40“ | DIN 11851 Male Part | DN 40 | EPDM | x |  |
|  | 01 | 5“ | TC | 1.5“ | Silicone |  | x |
|  | 01 | 5“ | TC | 1.5“ | EPDM |  | x |
|  | 01 | 10“ | TC | 1.5“ | Silicone |  | x |
|  | 01 | 10“ | TC | 1.5“ | EPDM |  | x |
|  | 01 | 20“ | TC | 1.5“ | Silicone |  | x |
|  | 01 | 20“ | TC | 1.5“ | EPDM |  | x |
|  | 01 | 30“ | TC | 1.5“ | Silicone |  | x |
|  | 01 | 30“ | TC | 1.5“ | EPDM |  | x |
|  | 01 | 40“ | TC | 1.5“ | Silicone |  | x |
|  | 01 | 40“ | TC | 1.5“ | EPDM |  | x |

| SPARE PARTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POS. | DESCRIPTION | MATERIAL | PART NO. | Q.TY |
| 1 | HOUSING BOWL | AISI 316L |  | 1 |
| 2 | CLAMP 3.5“ | AISI 304 |  | 1 |
| 4 | GASKET CLAMP 3.5“ | SILICONE |  | 1 |
|  | GASKET CLAMP 3.5“ | EPDM |  | 1 |

# Type 805 - Gas Multi-round Housings

## (5 bar Version)

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | FASTENING SYTEM | De [mm] | C [mm] | K [mm] | CONNECTIONS DN max /  max mm | TOTAL VOLUME [L] | PED Category |
| 03 x 10“ | V V R | 219 273 320 | 829 | 443 511 515 | DN 100 /  114.3mm DN 125 /  139.7mm DN 125 /  139.7mm | 33 | I |
| 03 x 20“ |  |  | 1079 |  |  | 42 | II |
| 05 x 10“ |  |  | 875 |  |  | 46 | II |
| 05 x 20“ |  |  | 1125 |  |  | 61 | II |
| 08 x 10“ |  |  | 935 |  |  | 59 | II |
| 08 x 20“ |  |  | 1175 |  |  | 82 | II |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Heigh t | Connection | Nominal Diameter | Gasket | With Vent u Drain BSPF 1/2“ female | With Vent u Drain TC 1.5“ | Standard |
|  | 03 | 10“ | FLANGE DIN EN 1092-1 | DN 100 | Silicone | x |  | x |
|  | 03 | 10“ | FLANGE DIN EN 1092-1 | DN 100 | EPDM | x |  |  |
|  | 03 | 20“ | FLANGE DIN EN 1092-1 | DN 100 | Silicone | x |  | x |
|  | 03 | 20“ | FLANGE DIN EN 1092-1 | DN 100 | EPDM | x |  |  |
|  | 05 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone | x |  | x |
|  | 05 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM | x |  |  |
|  | 05 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone | x |  | x |
|  | 05 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM | x |  |  |
|  | 08 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone | x |  | x |
|  | 08 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM | x |  |  |
|  | 08 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone | x |  | x |
|  | 08 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM | x |  |  |
|  | 03 | 10“ | FLANGE DIN EN 1092-1 | DN 100 | Silicone |  | x | x |
|  | 03 | 10“ | FLANGE DIN EN 1092-1 | DN 100 | EPDM |  | x |  |
|  | 03 | 20“ | FLANGE DIN EN 1092-1 | DN 100 | Silicone |  | x | x |
|  | 03 | 20“ | FLANGE DIN EN 1092-1 | DN 100 | EPDM |  | x |  |
|  | 05 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone |  | x | x |
|  | 05 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM |  | x |  |
|  | 05 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone |  | x | x |
|  | 05 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM |  | x |  |
|  | 08 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone |  | x | x |
|  | 08 | 10“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM |  | x |  |
|  | 08 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | Silicone |  | x | x |
|  | 08 | 20“ | FLANGE DIN EN 1092-1 | DN 125 | EPDM |  | x |  |
| OTHER SIZES ON REQUEST |  |  |  |  |  |  |  |  |

# Type 808 - industrial

- round - liquid housing

| TECHNICHAL DATA |  |
| --- | --- |
| NO. CARTRIDGED | 01 |
| CARTRIDGE SOCKET | 25 (Code 7) |
| CERTIFICATION | PED 2014/68/EU - art. 4 / sec. 3 |
| FLUID GROUP | GROUP 2 (not dangerous) |
| DESIGN PRESSURE 0°C-40°C | 8.0 bar |
| DESIGN PRESSURE 150°C | 6.2 bar |
| TEST PRESSURE (PT) | 10.5 bar |
| VESSEL MATERIAL | 316L / 1.4404 |
| GASKET | SILICONE |
| INTERNAL FINISHING | PICKLED |
| INTERNAL FINISHING | MECHANICAL POLISHING |
| DRAIN CONNECTION H + M | BSP 1/4“ FEMALE |
| VENT CONNECTION G | BSP 1/4“ MALE |
| DISTANCE L (ADAPTER 25 - CODE 7) | 110 mm |
| DISTANCE L (ADAPTER 21 - CODE 8) | 98 mm |

| DIMENSIONAL DATA |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (Y) | C mm | D mm | E mm | X mm | Y mm | VOLUME liters (V) | BOWL WEIGHT (kg) | TOTAL NET WEIGHT (kg) |
| 01 x 10“ | 572 | 382 | 300 | 440 | 320 | 1.8 | 1.1 | 4 |
| 01 x 20“ | 827 | 637 | 554 | 695 | 575 | 3.3 | 1.9 | 4.8 |
| 01 x 30“ | 1082 | 892 | 808 | 950 | 830 | 4.8 | 2.7 | 5.6 |
| 01 x 40“ | 1337 | 1147 | 1062 | 1205 | 1085 | 6.3 | 3.5 | 6.4 |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Adapter | Version | Connection Inlet u Outlet | Gasket |
|  | 01 | 10“ | 25 (Code 7) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 20“ | 25 (Code 7) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 30“ | 25 (Code 7) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 10“ | 25 (Code 7) | ADVANCED | BSP 1“ FEMALE | Silicone |
|  | 01 | 20“ | 25 (Code 7) | ADVANCED | BSP 1“ FEMALE | Silicone |
|  | 01 | 30“ | 25 (Code 7) | ADVANCED | BSP 1“ FEMALE | Silicone |
|  | 01 | 10“ | 21 (Code 8) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 20“ | 21 (Code 8) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 30“ | 21 (Code 8) | ADVANCED | BSP 3/4“ FEMALE | Silicone |
|  | 01 | 10“ | 21 (Code 8) | ADVANCED | BSP 1“ FEMALE | Silicone |
|  | 01 | 20“ | 21 (Code 8) | ADVANCED | BSP 1“ FEMALE | Silicone |
|  | 01 | 30“ | 21 (Code 8) | ADVANCED | BSP 1“ FEMALE | Silicone |

# Type 800 - Vent Housings

## (Pressureless)

| DIMENSIONAL DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CARTRIDGE x HEIGHT (N) | A [mm] | B [mm] |  | C [mm] (height needed for removing the bell over the cartris- ges) | D [mm] | E [mm] | HANDLES |
|  |  | CLAMP ASME BPE | DIN 11851 LINER |  |  |  |  |
| 01 x 10“ | 315 | 43 | 59 | 300 | 88.9 | 130 | NO |
| 01 x 20“ | 565 | 43 | 59 | 550 | 88.9 | 130 | NO |
| 01 x 30“ | 815 | 43 | 59 | 800 | 88.9 | 130 | NO |
| 03 x 10“ | 351 | 97 | 127 | 300 | 219 | 400 | NO |
| 03 x 20“ | 601 | 97 | 127 | 550 | 219 | 400 | YES |
| 03 x 30“ | 851 | 97 | 127 | 800 | 219 | 400 | YES |
| 05 x 10“ | 345 | 106 | 133 | 300 | 219 | 400 | NO |
| 05 x 20“ | 595 | 106 | 133 | 550 | 219 | 400 | YES |
| 05 x 30“ | 845 | 106 | 133 | 800 | 219 | 400 | YES |
| 08 x 10“ | 349 | 120 | 147 | 300 | 273 | 440 | NO |
| 08 x 20“ | 599 | 120 | 147 | 550 | 273 | 440 | YES |
| 08 x 30“ | 849 | 120 | 147 | 800 | 273 | 440 | YES |

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection Type Nominal Diameter / Size |  |
|  |  |  | Clamp ASME BPE | DIN 11851 Liner |
|  | 01 | 10“ | 2“ (50.8mm x 1.65mm) |  |
|  | 01 | 20“ | 2“ (50.8mm x 1.65mm) |  |
|  | 01 | 30“ | 2“ (50.8mm x 1.65mm) |  |
|  | 03 | 10“ | 3“ (76.2mm x 1.65mm) |  |
|  | 03 | 20“ | 3“ (76.2mm x 1.65mm) |  |
|  | 03 | 30“ | 3“ (76.2mm x 1.65mm) |  |
|  | 05 | 10“ | 4“ (101.2mm x 2.11mm) |  |
|  | 05 | 20“ | 4“ (101.2mm x 2.11mm) |  |
|  | 05 | 30“ | 4“ (101.2mm x 2.11mm) |  |
|  | 08 | 10“ | 4“ (101.2mm x 2.11mm) |  |
|  | 08 | 20“ | 4“ (101.2mm x 2.11mm) |  |
|  | 08 | 30“ | 4“ (101.2mm x 2.11mm) |  |
|  | 01 | 10“ |  | DN 50 |
|  | 01 | 20“ |  | DN 50 |
|  | 01 | 30“ |  | DN 50 |
|  | 03 | 10“ |  | DN 80 |
|  | 03 | 20“ |  | DN 80 |
|  | 03 | 30“ |  | DN 80 |
|  | 05 | 10“ |  | DN 100 |
|  | 05 | 20“ |  | DN 100 |
|  | 05 | 30“ |  | DN 100 |
|  | 08 | 10“ |  | DN 100 |
|  | 08 | 20“ |  | DN 100 |
|  | 08 | 30“ |  | DN 100 |
| OTHER SIZES ON REQUEST |  |  |  |  |

FILTER SERIES 800 - VENT JUMBO HOUSING	Page 1 | 1 HOUSING STYLE - I-TYPE - Series SAR

| TECHNICAL DATA |  |
| --- | --- |
| Material | 316L |
| Internal Surface | Ra 0.8μm EP |
| External Surface | RA 1.6μm EP |
| Sartorius Adapter | 40 |
| Qty. cartridge | 1 |
| Height | 10“, 20“ u 30“ |
| Connector | DIN 11851 |

Parts not in contact with the Medium are made by Stainless Steel 304. Jumbo vent housings are delivered with Certificates of the materials.

| ORDER CODE AND TECHNICAL EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| Order-no. | No. of Cartridges | Bowl Height | Connection Type DIN 11851 Liner |
|  | 01 | 10“ | DN 100 (104x2) |
|  | 01 | 20“ | DN 100 (104x2) |
|  | 01 | 30“ | DN 100 (104x2) |